CONDRE CD/DVD TRACER PRO

OPERATOR’S MANUAL

CONDRE, INC.

Phone: Toll Free: 1-866-500-3472

Direct: (952) 294-4900

Technical Support: (952) 253-6843

FAX: (952) 937-8365

Email: techsupport@condre.com

Headquarters Location:

15151 Technology Drive

Eden Prairie, MN 55344

Document History:  Edition 1.2 Rev.1 © Copyright 2002-2003, All rights reserved.

Notices:  The information in this document is subject to change without notice.

NO WARRANTY OF ANY KIND IS MADE WITH REGARD TO THIS MATERIAL, INCLUDING, BUT NOT LIMITED TO, THE IMPLIED WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE. No liability is assumed for errors contained herein or for incidental or consequential damages in connection with the furnishing, performance, or use of this material. This document contains proprietary information which is protected by copyright. All rights are reserved. No part of this document may be photocopied, reproduced, or translated into another language without prior written consent.

CAUTION!

TO PREVENT RISK OF FIRE OR ELECTRIC SHOCK HAZARD, ENSURE THAT THE UNIT IS NOT EXPOSED TO RAIN, MOISTURE, PPING OR SPLASHING AND THAT NO OBJECTS FILLED WITH LIQUIDS, SUCH AS VASES ARE PLACED ON THE UNIT.

TO REDUCE THE RISK OF ELECTRIC SHOCK, DO NOT REMOVE EXTERIOR PANELS. NO USER SERVICABLE PARTS ARE INSIDE. DOING SO WILL VOID YOUR WARRANTY. REFER SERVICING TO QUALIFIED SERVICE PERSONNEL. OPERATE THE UNIT WITH ONLY THE PROPER ELECTRICAL SPECIFICATIONS AS LABELED IN THE TECHNICAL SPECIFICATIONS OR ON THE POWER SUPPLY.

WARNING!

DO NOT INSTALL THIS UNIT IN A CLOSET, CABINET OR IN ANOTHER CONFINED SPACE. ENSURE THAT THE UNIT IS WELL VENTILATED. TO PREVENT RISK OF ELECTRIC SHOCK OR FIRE HAZARD DUE TO OVERHEATING, ENSURE THAT CURTAINS AND ANY OTHER MATERIALS DO NOT OBSTRUCT THE VENTILATION OPENINGS.

This document is © Copyright Condre, Inc.

Any reproduction without express permission from Condre, Inc. is strictly prohibited. Stylus Standalone products are trademarks of Condre, Inc. All other trademarks are the property of their respective companies. All rights reserved.

Table of Contents

Manufacturer Information ……………………………..	2

Important Safety Instructions 	………………………..	4

User Memo ……………………………………………….	4

Quick Start Guide  ………………………………………	5

Getting Started  ………………………………………….	6

Thank You ……………………………………….	6

Unpacking and Inspection ……………………..	6

Choosing a Location  ……………………………	6

Introduction  ………………………………………………	7

Changing the Hard Drive Partition  …………………..	8

USB  2.0 Option ………………………………………….	10

Menu Items  ………………………………………………	10

Primary Menu   ………………………………….	10

Secondary Menu   ………………………………	11

Setup Menu  …………………………………….	14

System Defaults   ……………………………………….	17

Troubleshooting  ……………………………………….	18

Limited Warranty   ……………………………………….	21

Important Safety instructions

Please be aware of the following installation and operation precautions.

| 1. Read these instructions - All the safety and operating instructions should be read before the appliance is operated. 2. Keep these instructions - The safety and operating instructions should be retained for future reference. 3. Heed all warnings - All warnings on the appliance and in the operating instructions should be adhered to. 4. Follow all instructions - All operating and use instructions should be followed. 5. Do not use this apparatus near water - for example, near a bathtub, washbowl, kitchen sink, laundry tub, in a wet basement or near a swimming pool, etc. 6. Clean only with a damp cloth. The appliance should be cleaned only as recommended by the manufacturer. 7. Install in accordance with the manufacturers instructions. Do not block any of the ventilation openings. For example, the appliance should not be situated on a bed, sofa, rug, or similar surface or placed in a built-in installation, such as a bookcase or cabinet that may impede the flow of air through the ventilation openings. 8. Do not install near any heat sources such as radiators, heat registers, stoves, or other apparatus (including computers) that produce heat. | 9. Do not defeat the safety purpose of the grounding-type plug. A grounding type plug has two blades and a third grounding prong. The third prong are provided for your safety. When the provided plug does not fit into your outlet, consult an electrician for replacement of the obsolete outlet. 10. Protect the power cord from being walked on or pinched particularly at plugs, convenience receptacles, and the point where they exit from the apparatus. 11. Only use attachments/accessories specified by the manufacturer. 12. Use only with a cart, stand, tripod, bracket, or table specified by the manufacturer, or sold with the apparatus. When a cart is used, use caution when moving the cart/apparatus combination to avoid injury from tip-over. 13. Unplug this apparatus during lightning storms or when unused for long periods of time. 14. Refer all servicing to qualified service personnel. Servicing is required when the apparatus has been damaged in any way, such as power-supply cord or plug is damaged, liquid has been spilled or objects have fallen into the apparatus, the apparatus has been exposed to rain or moisture, does not operate normally, or has been dropped. |
| --- | --- |

User Memo

Model Number:

Serial Number:

Date of Purchase:

Vendor Name:

Vendor Phone:

Vendor Address:

Quick Start Guide

1.)  When you first turn on the machine it will power up and start in Copy Mode.

2.)  To load a CD/DVD into your Condre system, push the “Mode” button four times until the display reads Load Mode.

3.)  Open the top drive, place your master CD/DVD in the tray, and close it. The system will automatically analyze the CD/DVD in the drive and proceed to load it to the HD.

4.)  Once the load is complete, the system will flash between Load and Pass. It will also eject the master CD/DVD.

5.)  Remove your master CD/DVD from the drive.

6.)  Push the “Mode” Button six times to bring the Load Mode back to Copy Mode. The display should have some new text on it. This text is explained in the Understanding Your Display section of the users manual.

7.)  Open the remaining drives of your Condre system and place a blank CD/DVD in every drive. Close each drive as you put a blank CD/DVD into them.

8.)  The display will now count up from 1 Start, 2 Start, 3 Start, and so on. Depending on the amount of drives your Condre system has. There is no need to press “Go” the system will automatically start if all drives have a blank CD/DVD in them.

9.)  Once the copy is completed the system will eject the drives. If you placed in a bad CD/DVD by accident, the system will flash Pass # Fail #. The good CD/DVD’s will be ejected and the bad CD/DVD’s will be kept in their drive.

10.) Remove the good CD/DVD’s label them and put them away. If you got a bad burn on one CD/DVD, toss that CD/DVD away.

11.) If you would like to make more copies of the same CD/DVD. Repeat steps 7 threw 10, until you have the desired amount. If you would like to make copies of an entirely new CD/DVD, repeat steps 2 threw 10.

* To reset factory defaults on your unit simply power the unit down.  Press and hold the “Mode” button for 15 seconds while the unit is powering up.  You will see the display go through a series of tests and boot the “Copy” mode screen.

Getting Started

Thank you...

...for choosing a Condre Standalone Tower  Duplicating System from Condre, Inc.

Condre Duplicators are second to none in speed and capacity and are backed by an intuitive interface design. We hope it will be a valuable addition to your business for years to come.

Unpacking and Inspection

When unpacking your unit, inspect the carton and packaging material for damage. Damages that may have occurred during shipping could cause alignment and operation errors in your unit. Any damages found upon receiving your unit should be reported to the dealer you purchased it from immediately.

Important!: Keep your packaging! In the event you need to ship the unit back to the manufacturer, the product must be shipped in the manner in which it was received. Other containers are not satisfactory and will void your warranty! If the container is lost or destroyed, another may be purchased from the manufacturer at a cost of 100$ plus shipping. Contact your vendor for details.

Choosing a Location

Placing your unit in an unsuitable location can cause a number of problems. Many problems can be avoided by following these guidelines.

- The unit must be placed on a flat, level surface. A tilt to the unit’s base can cause errors in calibration during use. The surface must also be relatively stable and resistant to bumps and jarring.

- The surface the unit is placed on must be able to hold 100+ pounds and be resistant to bowing under such weight.

- Place the unit in a location with adequate air circulation. Closets or cabinets are not suitable as heat build-up can cause the unit to malfunction. Likewise, do not place the unit on or near any heat sources such as heating vents or server racks.

- The unit must not be placed in an area with direct sunlight, excessive dust, mechanical vibration or shock.

Introduction

The Tracer Pro Standalone Duplicator is a stand-alone duplicator that requires no computer connection to operate. Low cost, high performance, the Condre Tracer Pro Standalone series will fulfill your duplication requirements.

Especially designed for office applications, the Condre Tracer Pro Standalone Duplicator is based upon a controller with a large LCD display and simple two-button navigation, which make steps easy to follow during the duplication process. Should you encounter any difficulty, please refer to the "Troubleshooting" section (Page 23) or contact Condre Technical Support.

We strongly suggest you locate your Condre Tracer Pro in a dust free environment. Dirty discs may cause duplication rejects or even total failure. Do not place unnecessary items on the unit.

Important!: Make sure the voltage setting on the power supply is set correctly for the region in which you are located. Typically, the setting is 115V in the United States. Your unit should arrive pre-set to 115V.

For recordable media selection, we recommend you choose high-speed rated and certified brand name discs (i.e. CMC, Taiyo Yuden, Fuji,  Pioneer, Imation, etc.) and make sure to reserve some discs for possible troubleshooting issues in the future. Under no circumstance does Condre, Inc. encourage users to duplicate copyrighted or copy-protected materials.

Changing the Hard Drive Partition

Setup

<Mode               Go>

1. Hold down the MODE button for about 5 seconds and then release. The display should read:

2. Press the GO button to enter the Setup Menu. Press Go again until the display reads something like:

HDD Partition

[*] A               [Q]

Partitions are represented by letters of the English alphabet. In the case above, the letter “A” is the currently selected partition. Press the MODE button to change the partition that the Stylus Standalone will copy from and load to.

In addition to hard drive partitions, there are Partition Groups. In the Setup Menu, press the Go button until the display reads:

HDD Partition Group

[*] A               [Q]

Partition Groups differ on the capacity of the hard drive installed in your unit. For example, if the capacity is 40GB, then HDD Partition Groups available might be A and B. If Group A is selected, partitions A-Z (900MBx26) will be available; while partition Group B divides the remaining space into partitions A-G (900MBx7). Press the MODE button to change the Partition Group. Press the GO button to confirm.

We have provided a table on the following page to keep track of images you have stored on your hard drive partitions.

Here we have provided a table for you to keep track of what you have stored on your hard drive partitions. You may wish to photocopy this for future use.

|  | Partition Group A |  | Partition Group B |
| --- | --- | --- | --- |
| A |  | A |  |
| B |  | B |  |
| C |  | C |  |
| D |  | D |  |
| E |  | E |  |
| F |  | F |  |
| G |  | G |  |
| H |  | H |  |
| I |  | I |  |
| J |  | J |  |
| K |  | K |  |
| L |  | L |  |
| M |  | M |  |
| N |  | N |  |
| O |  | O |  |
| P |  | P |  |
| Q |  | Q |  |
| R |  | R |  |
| S |  | S |  |
| T |  | T |  |
| U |  | U |  |
| V |  | V |  |
| W |  | W |  |
| X |  | X |  |
| Y |  | Y |  |
| Z |  | Z |  |

USB 2.0 Options

** Only for the DVD Tracer Pro Standalone **

On the back of each Condre DVD Tracer Pro Standalones there is a USB 2.0 Connector. This slot lets you connect the bottom drive of the Condre DVD Tracer Pro to your PC.

Items Needed:

- One open USB 2.0 slot on your PC.

- One USB cable (Supplied with unit).

- One Condre DVD Tracer Pro Standalone.

Using the USB Connection:

Have your PC on and booted up ready to go. Have the Condre DVD Tracer Pro Standalone plugged into the wall, and the USB swtich on the back of the machine switched to “ON”.

Turn on the Condre DVD Tracer Pro Standalone.  On your computer you should soon see a pop-up message saying New Device Detected.  If you open up “My Computer” on your PC you should now see a new DVD drive there.

You can now use this drive to create DVD’s with. It is recommended you use Record Now Max 4.5 or later. You can get this software from the dealer you purchased your Condre DVD Tracer Pro Standalone.

When You are done using this drive and ready to duplicate. Turn the USB switch back to “Off”, and restart the Condre DVD Tracer Pro Standalone.

Menu Items

Primary Menu:

After switching on your duplicator, pressing the MODE button will present you with a series of options...

Copy:  This is the primary function of the duplicator. When set to Copy, the duplicator will transfer the contents of a master disc or the current hard drive partition to any blank discs in the tower input bins.

Copy + Compare:  Like the “Copy” function, but followed by the Compare function (see “Compare”).

Emulate: Like the “Copy” function, but nothing is actually written to the blank discs. It is used for testing Master disc readability and hardware.

Emulate and Copy:  After emulation is completed and successful, the duplicator will begin writing the actual data to the discs.

Load:  Your unit has a hard drive; this function is used to load a Master disc onto its current partition. A Master must first be loaded to the hard drive before the Copy function can be used. You may also use the Batch Copy procedure to load masters to your Hard drive (Page 12).

Compare:  This function will compare bit-for-bit the source disc and destination discs and will report an error if the two sets of discs do not match. This function does not work with Audio or Video CDs (VCD). Audio or Video CDs will instead be verified (see “Verify”).

Quick Erase:  Data on CD-RW/DVD-RW will be erased.

Verify:  Verification compares the structure of the discs in any drive against the industry standard. This function is good for verifying that your source disc is a suitable Master and can be read in most standard devices.

Secondary Menu:

Holding down the MODE button for 5 seconds while in the Primary Menu will bring up the Secondary Menu. Press the MODE button to cycle through the options.

Edit

This function enables you to collect songs from different discs and copy the compilation to your blanks.

- Go to "Clear" function first to delete the data stored in the current hard drive.

Begin Track

[+] 0            [Quit]

Then switch to "Edit" and place your master disc into CD-ROM/DVD-ROM. The duplicator will show total XX tracks on LCD display and finally stop at: Press MODE switch to change the number of track that you wish to start with and then press Enter switch to quit. After quitting, the LCD display will stop at:

- Select end track, press Enter switch to quit.  i.e. No.1 Master disc: 2, 4, 6-8 Begin track 2, Enter to quit. End track 2, Enter to quit. Begin track 4, Enter to quit. End track 4, Enter to quit. Begin track 6, Enter to quit. End track 8, Enter to quit.

- Repeat the same procedure if more tracks will be selected. If no more tracks will be selected, press Enter switch directly at "Begin track" and press once again Enter at "End track" to exit.

- Automatically, duplicator will start loading those music tracks selected into the current partition of hard drive. When loading is completed, the master disc will be ejected automatically and LCD will display "Pass". You may change the master disc and run the Edit function again to gather more music tracks from different master discs.

- When collection is completed, switch the function to "Copy". Place blank discs into Input bin for copying.

Delete

Selecting "Delete" and pressing the GO button will delete the last track of a multi-track disc stored in the current partition of the hard drive.

Clear

Pressing GO will delete the contents of the currently selected partition.

Copy x1

This is a quick way to lower the speed of the writers to 1x. In some circumstances, this will reduce duplication errors.

Emulate x1

This will emulate the “Copy x1” function without actually writing any data to the blanks.

Copy x2

This is a quick way to lower the speed of the writers to 2x. In some circumstances, this will reduce duplication errors.

Emulate x2

This will emulate the “Copy x2” function without actually writing any data to the blanks.

Copy x4

This is a quick way to lower the speed of the writers to 4x. In some circumstances, this will reduce duplication errors.

Emulate x4

This will emulate the “Copy x4” function without actually writing any data to the blanks.

Copy x8

This is a quick way to lower the speed of the writers to 8x. In some circumstances, this will reduce duplication errors.

Emulate x8

This will emulate the “Copy x8” function without actually writing any data to the blanks.

Burn In

This function allows you to test if the duplicator is working properly. Please make sure a master disc is placed into the CD-ROM / DVD-ROM first.

Update

Under no circumstances should you attempt to update the firmware without express permission from the manufacturer or with firmware that is not Condre Tower-specific firmware provided by the manufacturer.

Load the disc containing the latest firmware data into a CD-ROM/DVD-ROM drive and the duplicator should find the data and update its firmware automatically. During updating, don't turn off the power of machine until the update is complete. Turning off the machine during an update may cause severe damage!

Setup Menu:

Holding down the MODE button for about 3 seconds then releasing, will bring up the Secondary Menu. Press the GO button once to activate the Setup Menu. We suggest you use the default settings during the whole process and not to make any changes unless you are familiar with these functions. In the event a setting is accidentally changed, a list of default settings can be found on Page 21.

Show Drives

The LCD will display all devices connected with the duplicator, i.e. CD-ROM / DVD-ROM, CD-writer / DVD-writer, and hard drive. Press the MODE button to scroll through the devices. Press GO to exit.

HDD Partition

You can store CD or DVD “images” on the hard drive in your Condre Tower. Images are stored on sections of the hard drive called partitions. When you load a new image to the hard drive, it will erase the image residing in the current partition. To store more then one image on the hard drive, you will need to select another partition. Here, you can change the current partition that the Condre Tower will load your master to. Partitions are identified by letters A-Z. Press the MODE button to change the current partition. Press GO to confirm.

HDD Partition Group

Partition groups differ on the capacity of the hard drive installed in your unit. For example, if the capacity is 30GB, then HDD Partition Groups available will be A and B. If Group A is selected, partitions A-Z (900MBx26) will be available; while partition Group B divides the remaining space into partitions A-G (900MBx7). Press the MODE button to change the Partition Group. Press GO to confirm.

CD to CD

Set "Yes" to copy directly from CD-RW / DVD-RW to another burner or set "No" to copy from selected partition of hard drive via the autoloader. Press the MODE button to change the setting. Press GO to confirm.

Start Writers

This controls the number of writers that will work in the autoloader. This should normally equal the number of writers installed. However, if you wish to only use 2 out of 4 writers for example, then you can select 2 here the remaining 2 will be ignored during duplication. Press the MODE button to change the setting. Press GO to confirm.

Write Method

Choose Disc-At-Once or Track-At-Once. Track-At-Once is normally used only when mixing multiple tracks via the controller. We recommend leaving this at Disc-At-Once. Press the MODE button to change the setting. Press GO to confirm.

Write Speed

Default setting is "Max", which means the writing speed will be the highest the drives in your unit support. In some cases, lowering the speed will ease duplication of troublesome masters. You may also lower the speed to allow use of older or slower-rated recordable media. Press the MODE button to change the setting. Press GO to confirm.

CD-ROM Drive

Allows selection of the CD-Rom reader. The default setting for this is Auto and should remain in Auto, as the Falcon-SC will not have CD-Rom drive installed. Press the MODE button to change the setting. Press GO to confirm.

Autoloader

This should remain set to “NO” at all times. Press the MODE button to change the setting. Press GO to confirm.

Grip Before Accept

This should remain set to “No” at all times. Press the MODE button to change the setting. Press GO to confirm.

Load Nonblank Disc

This setting allows the duplicator to load a master placed in the input bin to the hard drive for duplication. If this setting is not “Yes”, then the Condre Tower’s Batch Mode will not function correctly! This should remain set to “Yes”. Press the MODE button to change the setting. Press GO to confirm.

CD-DA Load Speed

When duplicating an Audio CD, the Stylus Standalone will automatically lower the read-speed to preserve quality. Setting this to an option other than “Auto” may increase read-in time, but may also introduce errors. It is recommended that this setting remain at “Auto. Press the MODE button to change the setting. Press GO to confirm.

Eject Disc

This should remain set to “Pass” at all times. Press the MODE button to change the setting. Press GO to confirm.

Buzzer

This turns on or off the buzzer tone in the duplicator. Press the MODE button to change the setting. Press GO to confirm.

CD+G

Changing this setting to “Yes” will allow the Condre Tower to duplicate CD+G (or “Karaoke”) discs if the installed writers support it. Press the MODE button to change the setting. Press GO to confirm.

Copy Comp Eject

This setting controls the ejection of writer's tray under the "Copy + Compare" function. This should remain set to “Auto” for the Copy + Compare feature to function correctly in the Stylus Standalone. Press the MODE button to change the setting. Press GO to confirm.

Read CD-DA Index 0

The setting controls the copying of data in Index 0 of Audio discs. Set to "Yes", the Condre Tower will copy the data, set to "No” will leave it empty. On Audio discs, Index 0 is sometimes used for album and artist info and track names. Press the MODE button to change the setting. Press GO to confirm.

Skip Read Error

This setting controls error skipping when reading in a master disc. If the setting is "No", when an error is detected during loading, the duplicator will stop loading and display a read-error message. If the setting is "Yes", there will be no message shown even there is an error in reading. You can use this function to copy discs with errors or non-standard track configuration. Press the MODE button to change the setting. Press GO to confirm.

Mode 2 Compare

This setting allows the unit to more properly compare Mode-2 formatted discs. Press the MODE button to change the setting. Press GO to confirm.

System Defaults

Should a setting in the Setup Menu be accidentally changed which causes the Condre CD/DVD Tracer Pro Standalone to behave incorrectly, you may need to change the settings back to their defaults. Here, we have provided the default settings found under the Setup Menu as they are when units arrive from the factory.

To get to the Setup Menu; after switching on your Condre CD/DVD Tracer Pro Standalone, wait for the unit to finish the startup procedure. When complete, hold down the MODE button for about 5 seconds and release. The display should read “Setup”. Press the GO button to enter the Setup Menu. Press the GO button again to cycle through the options.

Show Drives: (No setting)

HDD Partition: (User preference)

HDD Partition Group: (User preference)

CD to CD: No

Start Writers: Number of CD/DVD writers installed in unit.

Write Method: Disc-at-Once (DAO)

Write Speed: Max

CD-ROM Drive: Auto

Autoloader: No

Load Nonblank Disc: No

CD-DA Load Speed: Auto

Eject Disc: Pass

Buzzer: (User preference)

CD+G: No

Copy Comp Eject: Auto

Read CD-DA Index 0: No

Skip Read Error: No

Mode 2 Compare: No

Troubleshooting

There are no end-user serviceable components that can be replaced in the Condre CD/DVD Tracer Pro Standalone. Please contact your dealer for warranty repairs and service.

If the answer to your problem is not listed here, you may call Condre, Inc. Technical Support at: (952) 253-6843, 8:00AM to 5:30PM CST  Mon-Fri.

No LCD display.

- Please check if the power cord is connected properly and the utility power is supplied. Please check if the fan of power supply is rotating. If not, it might be broken.

Error message shown on LCD.

- Reboot the duplicator.

- Change master disc and try again.

CD Drive doesn’t read disc and shows error message.

- Check the format of master disc.

- Use other Master disc or other duplicator to test master.

Switches do not function.

- Task busy. Please wait.

- Restart unit.

- Faulty controller board. Call for Technical support

Copy completed but disc is not readable / executable.

- Use "Compare" to check.

- Test with another CD-ROM/DVD-ROM.

Failure rate is high.

- Lower writing speed and try again.

- Quality of blank disc is poor.

- Rated speed of discs does not match writers. Ensure discs meet writers rated speed.

- Update the latest BIOS version. See manufacturer for latest firmware updates.

Can't boot up or upgrade BIOS.

- Contact your dealer.

- Check if the BIOS program is correct.

Some functions are missing.

- Please reboot the unit and press Mode for 3 seconds at the same time.

Disc jam during operation.

- Two discs are being picked up.

- Ensure discs are not sticking together by fanning them within the input bin.

- Replace brand of discs with another type and retry.

Limited Warranty

Condre, Inc. () warrants that our s are free from defects in materials and workmanship for a period of twelve (12) months from the date appearing on your invoice    will provide remote technical support free of charge by phone or Email for 12 months after purchase of a  product through a dealer.  will, at it’s discretion, repair or replace the product. If failures occur, contact  Technical Support for assistance to determine a solution.  If a return for repair or replacement is determined to be necessary, a Return Material Authorization (RMA) number will be issued by Condre Technical Support .  Please refer to the RMA request procedure for details on return shipment requirements. Replacements requested before a return is received, known as “cross ship”, require the authorization of the dealer.

Attached printers are covered by the printer manufacturer warranty, refer to the printer documentation. Requests for refund must be handled by the dealer of the sale.

All products are shipped in specially designed packaging with inserts that have been designed and tested to ensure that our products can endure long-distance transport and still arrive in working order. We advise you to save your box and original packing materials in case you need to return the product(s) for any reason. If product(s) are returned without proper packaging, your warranty will be void. If you have lost or damaged your original packaging, another may be purchased from. Please call for the current price on the replacement packaging and shipping cost.

When you receive your product(s), please note the following:

- Verify all components are received according to your packing list;

- Inspect the shipping box for damage, if damage is found, inspect the product inside for damage.

- Notify technical support immediately if discrepancies are noted.

What is not covered:

This limited warranty provided by  does not cover:

- Products which have been subjected to abuse, accident, alteration, modification, tampering, negligence, misuse, faulty installation, lack of reasonable care, or if repaired or serviced by anyone without authorization of , Inc., or if the model or serial number has been altered, tampered with, defaced or removed.

- Normal maintenance and replacement of consumables.

-Losses due to theft, shipping damage other than initial or warranty replacement delivery.

-Accessories.

Notice: The products or software sold by , Inc. may be used to assist you in reproducing material in which you own the copyright or have obtained permission to copy from the copyright owner. Unless you own the copyright or have permission to copy from the copyright owner, you may be violating copyright law and be subject to payment of damages and other remedies. If you are uncertain about your rights you should contact your legal advisor.

If you experience trouble with any , Inc. product, please contact our Technical Support department by email or fax anytime,  by voice  between 8am and 5:30pm central time, Monday to Friday, except holidays.

CONDRE, INC.	    1-866-500-3472